Easy-to-Understand Notes: OSPF (Open Shortest Path First)

What is OSPF?

- OSPF: A link-state routing protocol designed to replace the older RIP protocol.

- Why OSPF is Better:

  - RIP uses hop count as the only metric, which is inefficient in large networks.

  - OSPF scales better and has faster convergence.

OSPF Key Features

- Classless: Supports CIDR and VLSM.

- Efficient: Sends updates only when routing changes occur, not periodically.

- Fast Convergence: Quickly adapts to network changes.

- Scalable: Supports both small and large networks using areas.

- Secure: Can use MD5 authentication to encrypt routing updates.

OSPF Administrative Distance (AD)

- AD of OSPF: 110 (more reliable than RIP or IS-IS).

- Connected Routes: AD = 0 (most preferred).

- Purpose of AD: Used by routers to choose the best path when multiple routing protocols provide a route to the same destination.

OSPF Data Structures

OSPF maintains three main databases:

- Adjacency Database: Builds the neighbor table.

- Link-State Database (LSDB): Builds the topology table.

- Forwarding Database: Creates the routing table.

OSPF Messages (5 Types of Packets)

- Hello Packet: Discovers neighbors and maintains adjacency.

- Database Description Packet: Summarizes LSDB.

- Link-State Request Packet: Requests missing LSDB info.

- Link-State Update Packet (LSU): Sends updated link-state info.

- Link-State Acknowledgment Packet: Confirms receipt of LSUs.

How OSPF Converges

- Exchange Hello Packets to establish neighbor relationships.

- Flood LSAs (Link-State Advertisements) throughout the network.

- Build the LSDB with received LSAs.

- Run the SPF Algorithm to calculate the best paths.

- Update the Routing Table with the best paths.

OSPF Metric: Cost

- Metric = Cost: Lower cost = better path.

- Cost Formula:

graphql

Copy code

Cost = Reference Bandwidth ÷ Interface Bandwidth

  - Default Reference Bandwidth = 100,000,000 (100 Mbps).

  - Command to Adjust Reference Bandwidth:

scss

Copy code

auto-cost reference-bandwidth [new_value]

  - Must be configured on all routers in the OSPF domain.

OSPF Passive Interfaces

- Used to stop OSPF updates from being sent out an interface (e.g., LAN interfaces).

- Command to Verify Passive Interfaces:

sql

Copy code

show ip protocols

OSPF Designated Router (DR) and Backup DR (BDR)

- DR: Manages LSAs in multi-access networks to reduce flooding.

- BDR: Backup for the DR if it fails.

- DROTHER: All other routers (neither DR nor BDR).

OSPF Router ID (RID)

- Router ID Selection Process:

  - Manually Configured RID (highest priority).

  - Highest Loopback Address.

  - Highest Active IP Address of a physical interface.

- Command to Set RID:

css

Copy code

router-id [ID]

- If no RID or loopback is configured, OSPF uses the highest active interface IP.

OSPF Areas

- Why Use Multiple Areas?

  - Reduces routing table size and LSDB size.

  - Improves network performance and scalability.

- Area 0 (Backbone Area): Required for multi-area OSPF implementations.

OSPF Timers

- Hello Interval: How often Hello packets are sent (default = 10 seconds).

- Dead Interval: Time to declare a neighbor as down if no Hello is received (default = 40 seconds).

OSPF States

- Down: No Hello received.

- Init: Hello packet received but no 2-way communication yet.

- 2-Way: Bi-directional communication established.

- Exchange: Exchanging LSDB summaries.

- Loading: Requesting missing LSAs.

- Full: Convergence complete; adjacency established.

Key OSPF Commands

- Enable OSPF:

arduino

Copy code

router ospf [process-id]

- Advertise Networks:

css

Copy code

network [IP] [wildcard-mask] area [area-id]

- Verify OSPF:

sql

Copy code

show ip ospf

show ip ospf neighbor

show ip route ospf

Summary

- OSPF = Efficient, Secure, and Scalable.

- Uses Cost as Metric for best path selection.

- DR/BDR reduces LSAs in multi-access networks.

- Converges Faster than distance-vector protocols like RIP.