Configuring Basic Single Area OSPF

Test connectivity

The routers should be able to ping one another, and each PC should be able to ping its default gateway. The PCs are unable to ping other PCs until OSPF routing is configured. Verify and troubleshoot if necessary.

Part1: Configure and Verify OSPF Routing

Configure OSPFv2 routing on all routers in the network and then verify that routing tables are updated correctly.

Step 1: Configure OSPF on R1

a. Use the router ospf command in global configuration mode to enable OSPF on R1.

R1(config)# router ospf 1

Note: The OSPF process id is kept locally and has no meaning to other routers on the network.

b. Configure the network statements for the networks on R1. Use an area ID of 0.

R1(config-router)# network 192.168.1.0 0.0.0.255 area 0

R1(config-router)# network 192.168.12.0 0.0.0.3 area 0

R1(config-router)# network 192.168.13.0 0.0.0.3 area 0

Step 2: Configure OSPF on R2 and R3

Use the router ospf command and add the network statements for the networks on R2 and R3.

Neighbour adjacency messages display on R1 when OSPF routing is configured on R2 and R3.

R1#

00:22:29: %OSPF-5-ADJCHG: Process 1, Nbr 192.168.23.1 on Serial0/0/0 from LOADING to FULL, Loading Done

R1#

00:23:14: %OSPF-5-ADJCHG: Process 1, Nbr 192.168.23.2 on Serial0/0/1 from LOADING to FULL, Loading Done

R1#

Step 3: Verify OSPF neighbors and routing information. a. Issue the show ip ospf neighbor command to verify that each router lists the other routers in the network as neighbors.

R1# show ip ospf neighbor

Neighbor ID Pri State Dead Time Address Interface

192.168.23.2 0 FULL/ - 00:00:33 192.168.13.2 Serial0/0/1

192.168.23.1 0 FULL/ - 00:00:30 192.168.12.2 Serial0/0/0

b. Issue the show ip route command to verify that all networks display in the routing table on all routers.

R1# show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

192.168.1.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.1.0/24 is directly connected, GigabitEthernet0/0

L 192.168.1.1/32 is directly connected, GigabitEthernet0/0

O 192.168.2.0/24 [110/65] via 192.168.12.2, 00:32:33, Serial0/0/0

O 192.168.3.0/24 [110/65] via 192.168.13.2, 00:31:48, Serial0/0/1

192.168.12.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.12.0/30 is directly connected, Serial0/0/0

L 192.168.12.1/32 is directly connected, Serial0/0/0

192.168.13.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.13.0/30 is directly connected, Serial0/0/1

L 192.168.13.1/32 is directly connected, Serial0/0/1

192.168.23.0/30 is subnetted, 1 subnets

O 192.168.23.0/30 [110/128] via 192.168.12.2, 00:31:38, Serial0/0

What command would you use to only see the OSPF routes in the routing table?

Step 4: Verify OSPF protocol settings.

The show ip protocols command is a quick way to verify vital OSPF configuration information. This information includes the OSPF process ID, the router ID, networks the router is advertising, the neighbours the router is receiving updates from, and the default administrative distance, which is 110 for OSPF.

R1# show ip protocols

*** IP Routing is NSF aware ***

Routing Protocol is "ospf 1"

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Router ID 192.168.13.1

Number of areas in this router is 1. 1 normal 0 stub 0 nssa

Maximum path: 4

Routing for Networks:

192.168.1.0 0.0.0.255 area 0

192.168.12.0 0.0.0.3 area 0

192.168.13.0 0.0.0.3 area 0

Routing Information Sources:

Gateway Distance Last Update

192.168.23.2 110 00:19:16

192.168.23.1 110 00:20:03

Distance: (default is 110)

Step 5: Verify OSPF process information.

Use the show ip ospf command to examine the OSPF process ID and router ID. This command displays the OSPF area information, as well as the last time the SPF algorithm was calculated.

R1# show ip ospf

Routing Process "ospf 1" with ID 192.168.13.1

Start time: 00:20:23.260, Time elapsed: 00:25:08.296

Supports only single TOS(TOS0) routes

Supports opaque LSA

Supports Link-local Signaling (LLS)

Supports area transit capability

Supports NSSA (compatible with RFC 3101)

Event-log enabled, Maximum number of events: 1000, Mode: cyclic

Router is not originating router-LSAs with maximum metric

Initial SPF schedule delay 5000 msecs

Minimum hold time between two consecutive SPFs 10000 msecs

Maximum wait time between two consecutive SPFs 10000 msecs

Incremental-SPF disabled

Minimum LSA interval 5 secs

Minimum LSA arrival 1000 msecs

LSA group pacing timer 240 secs

Interface flood pacing timer 33 msecs

Retransmission pacing timer 66 msecs

Number of external LSA 0. Checksum Sum 0x000000

Number of opaque AS LSA 0. Checksum Sum 0x000000

Number of DCbitless external and opaque AS LSA 0

Number of DoNotAge external and opaque AS LSA 0

Number of areas in this router is 1. 1 normal 0 stub 0 nssa

Number of areas transit capable is 0

External flood list length 0

IETF NSF helper support enabled

Cisco NSF helper support enabled

Reference bandwidth unit is 100 mbps

Area BACKBONE(0)

Number of interfaces in this area is 3

Area has no authentication

SPF algorithm last executed 00:22:53.756 ago

SPF algorithm executed 7 times

Area ranges are

Number of LSA 3. Checksum Sum 0x019A61

Number of opaque link LSA 0. Checksum Sum 0x000000

Number of DCbitless LSA 0

Number of indication LSA 0

Number of DoNotAge LSA 0

Flood list length 0

Step 6: Verify OSPF interface settings.

a. Issue the show ip ospf interface  command to display a summary of OSPF-enabled interfaces.

R1# show ip ospf interface

Interface PID Area IP Address/Mask Cost State Nbrs F/C

Se0/0/1 1 0 192.168.13.1/30 64 P2P 1/1

Se0/0/0 1 0 192.168.12.1/30 64 P2P 1/1

Gi0/0 1 0 192.168.1.1/24 1 DR 0/0

b. For a more detailed list of every OSPF-enabled interface, issue the show ip ospf interface command.

R1# show ip ospf interface

Serial0/0/1 is up, line protocol is up

Internet Address 192.168.13.1/30, Area 0, Attached via Network Statement

Process ID 1, Router ID 192.168.13.1, Network Type POINT_TO_POINT, Cost: 64

Topology-MTID Cost Disabled Shutdown Topology Name

0 64 no no Base

Transmit Delay is 1 sec, State POINT_TO_POINT

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

oob-resync timeout 40

Hello due in 00:00:01

Supports Link-local Signaling (LLS)

Cisco NSF helper support enabled

IETF NSF helper support enabled

Index 3/3, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 1, Adjacent neighbor count is 1

Adjacent with neighbor 192.168.23.2

Suppress hello for 0 neighbor(s)

Serial0/0/0 is up, line protocol is up

Internet Address 192.168.12.1/30, Area 0, Attached via Network Statement

Process ID 1, Router ID 192.168.13.1, Network Type POINT_TO_POINT, Cost: 64

Topology-MTID Cost Disabled Shutdown Topology Name

0 64 no no Base

Transmit Delay is 1 sec, State POINT_TO_POINT

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

oob-resync timeout 40

Hello due in 00:00:03

Supports Link-local Signaling (LLS)

Cisco NSF helper support enabled

IETF NSF helper support enabled

Index 2/2, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 1, Adjacent neighbor count is 1

Adjacent with neighbor 192.168.23.1

Suppress hello for 0 neighbor(s)

GigabitEthernet0/0 is up, line protocol is up

Internet Address 192.168.1.1/24, Area 0, Attached via Network Statement

Process ID 1, Router ID 192.168.13.1, Network Type BROADCAST, Cost: 1

Topology-MTID Cost Disabled Shutdown Topology Name

0 1 no no Base

Transmit Delay is 1 sec, State DR, Priority 1

Designated Router (ID) 192.168.13.1, Interface address 192.168.1.1

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

oob-resync timeout 40

Hello due in 00:00:01

Supports Link-local Signaling (LLS)

Cisco NSF helper support enabled

IETF NSF helper support enabled

Index 1/1, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 0, maximum is 0

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 0, Adjacent neighbor count is 0

Suppress hello for 0 neighbor(s)

Step 7: Verify end-to-end connectivity.

Each PC should be able to ping the other PCs in the topology. Verify and troubleshoot if necessary.

Part 2: Change Router ID Assignments

The OSPF router ID is used to uniquely identify the router in the OSPF routing domain. Cisco routers derive the router ID in one of three ways and with the following precedence:

1) IP address configured with the OSPF router-id command, if present

2) Highest IP address of any of the router’s loopback addresses, if present

3) Highest active IP address on any of the router’s physical interfaces

Because no router IDs or loopback interfaces have been configured on the three routers, the router ID for each router is determined by the highest IP address of any active interface.

In Part 2, you will change the OSPF router ID assignment using loopback addresses. You will also use the router-id command to change the router ID.

Step 1: Change router IDs using loopback addresses

a. Assign an IP address to loopback 0 on R1.

R1(config)# interface lo0

R1(config-if)# ip address 1.1.1.1 255.255.255.255

R1(config-if)# end

b. Assign IP addresses to Loopback 0 on R2 and R3. Use IP address 2.2.2.2/32 for R2 and 3.3.3.3/32 for R3.

c. Save the running configuration to the startup configuration on all three routers!!! Copy run start!!

d. You must reload the routers in order to reset the router ID to the loopback address. Issue the reload command on all three routers. Press Enter to confirm the reload.

e. After the router completes the reload process, issue the show ip protocols command to view the new router ID.

R1# show ip protocols

*** IP Routing is NSF aware ***

Routing Protocol is "ospf 1"

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Router ID 1.1.1.1

Number of areas in this router is 1. 1 normal 0 stub 0 nssa

Maximum path: 4

Routing for Networks:

192.168.1.0 0.0.0.255 area 0

192.168.12.0 0.0.0.3 area 0

192.168.13.0 0.0.0.3 area 0

Routing Information Sources:

Gateway Distance Last Upd

Gateway Distance Last Update

3.3.3.3 110 00:01:00

2.2.2.2 110 00:01:14

Distance: (default is 110)

f. Issue the show ip ospf neighbor command to display the router ID changes for the neighboring routers.

R1# show ip ospf neighbor

Neighbor ID Pri State Dead Time Address Interface

3.3.3.3 0 FULL/ - 00:00:35 192.168.13.2 Serial0/0/1

2.2.2.2 0 FULL/ - 00:00:32 192.168.12.2 Serial0/0/0

R1#

Step 2: Change the router ID on R1 using the router-id command.

The preferred method for setting the router ID is with the router-id command.

a. Issue the router-id 11.11.11.11 command on R1 to reassign the router ID. Notice the informational message that appears when issuing the router-id command.

R1(config)# router ospf 1

R1(config-router)# router-id 11.11.11.11

Remember to save configuration!!!

Reload or use "clear ip ospf process" command, for this to take effect

R1(config)# end

b. You will receive an informational message telling you that you must either reload the router or use the clear ip ospf process command for the change to take effect. Issue the clear ip ospf process command on all three routers. Type yes to reply to the reset verification message, and press ENTER.

c. Set the router ID for R2 to 22.22.22.22 and the router ID for R3 to 33.33.33.33. Then use clear ip ospf process command to reset ospf routing process.

d. Issue the show ip protocols command to verify that the router ID changed on R1.

R1# show ip protocols

*** IP Routing is NSF aware ***

Routing Protocol is "ospf 1"

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Router ID 11.11.11.11

Number of areas in this router is 1. 1 normal 0 stub 0 nssa

Maximum path: 4

Routing for Networks:

192.168.1.0 0.0.0.255 area 0

192.168.12.0 0.0.0.3 area 0

192.168.13.0 0.0.0.3 area 0

Passive Interface(s):

GigabitEthernet0/1

Routing Information Sources:

Gateway Distance Last Update

33.33.33.33 110 00:00:19

22.22.22.22 110 00:00:31

3.3.3.3 110 00:00:41

2.2.2.2 110 00:00:41

Distance: (default is 110)

e. Issue the show ip ospf neighbor command on R1 to verify that new router ID for R2 and R3 is listed.

R1# show ip ospf neighbour

Neighbor ID Pri State Dead Time Address Interface

33.33.33.33 0 FULL/ - 00:00:36 192.168.13.2 Serial0/0/1

22.22.22.22 0 FULL/ - 00:00:32 192.168.12.2 Serial0/0/0